Abstract

Renewable energy microgrid design is a complex multi-objective optimization problem requiring simultaneous minimization of installation cost, operating cost, and energy loss, while maximizing renewable energy utilization, grid reliability, and load satisfaction. Traditional planning methods rely on manual calculations or deterministic optimization techniques, which are time-consuming and fail to find globally optimal solutions under varying environmental and load conditions.

EcoGridAI is an AI-powered software platform that addresses this challenge by employing two swarm intelligence algorithms — Artificial Bee Colony (ABC) and Particle Swarm Optimization (PSO) — to optimize renewable energy resource dispatch, battery sizing, and grid import limits on a real IEEE 33-bus radial distribution network. The platform processes real measured data from Data1.csv (hourly kW measurements) and Data2.csv (normalized per-unit profiles), with bus ratings of PV at Bus 13 (851.5 kW), PV at Bus 30 (1157.6 kW), and Wind at Bus 6 (2097.97 kW).

The system evaluates performance across 10 operating scenarios — Normal Operation, Peak Summer, Battery Failure, Grid Outage, Low Renewable, High Renewable Surplus, Night Peak, Partial Wind Failure, Partial PV Failure, and Extreme Stress — and presents results through an interactive Web dashboard with convergence curves, dispatch profiles, cost breakdowns, and a Capacity Recommendation Engine. Shannon Entropy Weight Method is applied to compute algorithm-specific objective weights for ABC and PSO independently, replacing static equal weighting.

Results demonstrate that PSO converges in 3–6 iterations (significantly faster than ABC at 9–48 iterations), while both algorithms outperform the random baseline across all 10 scenarios. Under normal operation, the system achieves 93.87% load satisfaction with the existing 33-bus installation, identifying a 915 kW capacity gap at hour 19 — a key engineering insight for future expansion planning.

Keywords: Microgrid Optimization, Artificial Bee Colony, Particle Swarm Optimization, IEEE 33-Bus, Shannon Entropy Weights, Swarm Intelligence, Renewable Energy, Python.

# Table of Contents

Bonafide Certificate

Declaration

Acknowledgement

Abstract

List of Figures

List of Tables

1. Introduction

1.1 Overview

1.2 Problem Statement

1.3 Motivation

1.4 Objectives

1.5 Scope

2. Background Study

2.1 Renewable Energy Microgrids

2.2 IEEE 33-Bus System

2.3 Swarm Intelligence

2.4 Artificial Bee Colony (ABC)

2.5 Particle Swarm Optimization (PSO)

3. Methodology

3.1 Decision Variables

3.2 Fitness Function

3.3 Shannon Entropy Weights

3.4 ABC Algorithm

3.5 PSO Algorithm

3.6 Priority Dispatch

4. System Architecture

4.1 Module Overview

4.2 Real Data Pipeline

4.3 Scenario Definitions

5. Results and Discussion

5.1 Baseline Pipeline Results

5.2 Real Data Pipeline Results

5.3 Algorithm Comparison

5.4 Capacity Recommendations

6. Dashboard & Visualization

7. Conclusions and Future Scope

8. References

# Chapter 1: Introduction

## 1.1 Overview

EcoGridAI is an AI-powered software platform designed to automate the planning and optimization of renewable energy microgrids. The system intelligently determines the optimal dispatch fractions and storage sizing for renewable energy resources — solar photovoltaic (PV) panels, wind turbines, battery energy storage systems, and utility grid connections — on a real IEEE 33-bus radial distribution network.

Instead of manual engineering calculations, EcoGridAI uses two powerful swarm intelligence algorithms — Artificial Bee Colony (ABC) and Particle Swarm Optimization (PSO) — to search for the best possible operational configuration in minutes. The system processes real measured data from an actual distribution system and evaluates designs across 10 operating scenarios.

## 1.2 Problem Statement

Designing and operating a renewable energy microgrid involves several competing challenges:

• Multi-objective conflict: minimizing cost conflicts with maximizing reliability and renewable utilization. No single formula solves all simultaneously.

• Non-convex search space: millions of possible dispatch configurations. Gradient-based methods get trapped in local optima.

• Time-varying conditions: solar energy available only 8 hours/day; wind varies hourly; load peaks at hour 19 (3715 kW). Any design must perform across all 24 hours.

• Real system capacity gap: the existing IEEE 33-bus installation (PV: 2009.1 kW + Wind: 2097.97 kW) achieves only 93.87% load satisfaction under normal conditions — a 915 kW gap at hour 19.

• Multiple failure scenarios: the system must remain operational during battery failure, grid outage, and extreme stress conditions.

## 1.3 Motivation

The motivation for EcoGridAI arises from three directions:

Energy Access Gap: Over 770 million people worldwide lack reliable electricity. Microgrids are among the most promising solutions for rural and off-grid communities. Intelligent planning tools are critical to making these systems economically viable.

Renewable Transition: The global shift from fossil fuels to solar and wind energy demands optimization tools that handle the variability and unpredictability of renewable sources.

Research Gap: While ABC and PSO have been applied to energy problems individually, very few studies provide a fair, standardized comparison of both algorithms on the same microgrid problem with multiple objectives, real measured data, and multiple failure scenarios.

## 1.4 Objectives

The specific objectives of this project are:

• To process and utilize real measured data from the IEEE 33-bus distribution system (Data1.csv and Data2.csv).

• To implement and compare Artificial Bee Colony (ABC) and Particle Swarm Optimization (PSO) algorithms on the microgrid optimization problem.

• To develop a Shannon Entropy Weight Method that computes algorithm-specific objective weights from actual output distributions.

• To benchmark both algorithms across 10 diverse operating scenarios.

• To build an interactive Web dashboard with KPI cards, convergence curves, dispatch profiles, and a Capacity Recommendation Engine.

• To quantify the 33-bus system capacity gap and provide actionable expansion recommendations.

## 1.5 Scope

EcoGridAI is a software-only simulation platform using real measured data. It does not interface with physical hardware. The optimization targets dispatch fractions of existing installed assets and battery/grid sizing. Physical installation planning (greenfield mode) is identified as future work.

# Chapter 2: Background Study

## 2.1 Renewable Energy Microgrids

A microgrid is a small, localized energy system that can operate independently or connected to the main utility grid. Unlike the national power grid, a microgrid serves a specific area — a campus, hospital, or industrial facility. EcoGridAI models the IEEE 33-bus radial distribution network, a standard benchmark used globally in power systems research.

The 33-bus system consists of 33 connection points forming a radial (branch) topology. In this project, PV generators are connected at Bus 13 (851.5 kW) and Bus 30 (1157.6 kW), and a wind turbine is connected at Bus 6 (2097.97 kW). The system peak load is 3715.0 kW at hour 19.

## 2.2 IEEE 33-Bus System Architecture

The network topology:

• Bus 1 (Substation): Main grid connection point. Acts as slack bus.

• Bus 6 (Wind): Wind turbine rated at 2097.97 kW connected at this bus.

• Bus 13 (PV): PV array rated at 851.5 kW connected at this bus.

• Bus 30 (PV): PV array rated at 1157.6 kW connected at this bus.

• Cable Loss Factor: Computed using bus distance from Bus 1. Loss factors range from 1.00 (Bus 1) to 1.15 (Bus 33). Bus 6 Wind has highest cable loss due to distance.

## 2.3 Swarm Intelligence

Swarm intelligence is a field of AI inspired by the collective behavior of natural systems — bee colonies, bird flocks, fish schools. These systems achieve intelligent behavior through simple local rules followed by many agents. Swarm algorithms are metaheuristic — they do not require mathematical derivatives or problem-specific knowledge. They maintain a population of solutions and explore the search space in parallel, avoiding local optima.

## 2.4 Artificial Bee Colony (ABC)

The ABC algorithm was proposed by Dervis Karaboga in 2005, inspired by the foraging behavior of honeybee swarms. Three bee roles:

• Employed Bees: Each explores the neighborhood of its food source (solution) using: v_ij = x_ij + φ_ij × (x_ij − x_kj), where φ ∈ [−1, 1].

• Onlooker Bees: Select food sources probabilistically based on quality: P_i = fitness_i / Σfitness_j.

• Scout Bees: Abandon exhausted sources (after "limit" trials) and discover new random solutions.

Parameters: Colony Size = 30, Limit = SN × D, Max Iterations = 100, Seed = 42.

## 2.5 Particle Swarm Optimization (PSO)

PSO was introduced by Kennedy and Eberhart in 1995, inspired by bird flocking. Each particle has a position (solution) and velocity. Update equations:

• Velocity: v_i(t+1) = w·v_i(t) + c1·r1·(pBest_i − x_i) + c2·r2·(gBest − x_i)

• Position: x_i(t+1) = x_i(t) + v_i(t+1)

• Inertia decay: w = 0.9 → 0.4 (linearly over iterations — explore early, exploit late)

Parameters: Swarm Size = 30, c1 = c2 = 2.0, w_max = 0.9, w_min = 0.4, Max Iterations = 100, Seed = 43.

# Chapter 3: Methodology

## 3.1 Decision Variables

The optimization problem uses 6 decision variables representing operational choices for existing fixed installations:

| Variable | Name | Range | Justification |
| --- | --- | --- | --- |
| x[0] | pv_bus13_fraction | 0.0 – 1.0 | Fraction of 851.5 kW PV at Bus 13 |
| x[1] | pv_bus30_fraction | 0.0 – 1.0 | Fraction of 1157.6 kW PV at Bus 30 |
| x[2] | wind_fraction | 0.0 – 1.0 | Fraction of 2097.97 kW Wind at Bus 6 |
| x[3] | battery_power_kw | 0 – 500 kW | Max charge/discharge rate |
| x[4] | battery_cap_kwh | 100 – 5000 kWh | 2× peak load for resilience |
| x[5] | grid_limit_kw | 0 – 2000 kW | Utility connection import limit |

## 3.2 Fitness Function

The multi-objective fitness function combines 5 normalized terms into a single scalar (lower = better):

F = w1·norm_capital + w2·norm_op + w3·norm_loss + w4·norm_unmet + w5·curtailment_penalty

Normalization constants: Capital = ₹150,000/day; Operational = ₹400,000/day; Loss = 5,000 kWh; Unmet = 10,000 kWh.

## 3.3 Shannon Entropy Weight Method

Instead of fixed equal weights, EcoGridAI computes algorithm-specific weights using Shannon Entropy Method applied to the actual output distributions across 10 scenarios:

• Step 1: Build decision matrix M (10 scenarios × 5 objectives) separately for ABC and PSO.

• Step 2: Normalize columns: p_ij = m_ij / Σm_ij

• Step 3: Compute entropy: E_j = −(1/ln n) × Σ(p_ij × ln p_ij)

• Step 4: Divergence: d_j = 1 − E_j

• Step 5: Weights: w_j = d_j / Σd_j

Result: Objectives that vary MORE across scenarios carry more information and receive higher weight. This produces different weights for ABC and PSO reflecting each algorithm's output distribution characteristics.

| Objective | Equal Weight | ABC Entropy | PSO Entropy | Winner | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Capital Cost | 0.20 | 0.0228 | 0.0297 | PSO | Low variation across scenarios |
| Operational Cost | 0.25 | 0.1191 | 0.1188 | Equal | Moderate variation |
| Energy Loss | 0.15 | 0.0226 | 0.0221 | Equal | Low variation across scenarios |
| Unmet Load | 0.30 | 0.4733 | 0.4733 | Equal | Highest — varies 0 to 8543 kWh |
| Curtailment | 0.10 | 0.3622 | 0.3561 | ABC | High — large surplus scenarios |

## 3.4 Priority Dispatch Order

At each hour t, the simulation follows the priority dispatch sequence:

• Priority 1 — Solar PV: Serves load first (zero marginal cost)

• Priority 2 — Wind: Serves remaining load (zero marginal cost)

• Priority 3 — Battery: Discharges if deficit remains; charges if surplus available

• Priority 4 — Grid Import: Fills remaining deficit up to grid_limit_kw

• Priority 5 — Unmet Load: If all sources insufficient, record interruption event

# Chapter 4: System Architecture

## 4.1 Module Overview

EcoGridAI consists of two independent pipelines:

| Resource_optimizer/ (Synthetic) | Resource_optimizer_real/ (Real Data) |
| --- | --- |
| planning_vector.py — synthetic bounds | real_data_loader.py — reads Data1.csv + Data2.csv |
| objective_function.py | bus_network.py — 33-bus topology + loss factors |
| abc_optimization.py | planning_vector.py — real capacity bounds |
| pso_optimization.py | dispatch_simulation_real.py — per-bus loss tracking |
| dispatch_simulation.py | entropy_weights.py — Shannon Entropy computation |
| scenarios.py — 10 scenarios | recommendation/engine.py — capacity recommendations |
| main.py — orchestrator | main_real.py — real data orchestrator |

## 4.2 Real Data Pipeline

Data1.csv contains 24 hourly rows of actual measured system data: Hour, Total Load kW, Commercial, Industry, Residential, PV kW, Wind kW. Bus ratings are embedded: PV@Bus13=851.5kW, PV@Bus30=1157.6kW, Wind@Bus6=2097.97kW. Peak load = 3715.0 kW at hour 19.

Data2.csv contains normalized per-unit profiles (values 0.0–1.0): Time, Load demand, Power from PV, Power from Wind. These profiles scale the actual rated capacities to produce hourly generation values.

## 4.3 Ten Scenario Definitions

| # | Scenario | Solar × | Wind × | Load × | Grid | Battery |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Normal Operation | 1.0 | 1.0 | 1.0 | Yes | Yes |
| 2 | Peak Summer | 1.1 | 1.0 | 1.3 | Yes | Yes |
| 3 | Battery Failure | 1.0 | 1.0 | 1.0 | Yes | No |
| 4 | Grid Outage | 1.0 | 1.0 | 1.0 | No | Yes |
| 5 | Low Renewable | 0.4 | 0.3 | 1.0 | Yes | Yes |
| 6 | High Renewable Surplus | 1.5 | 1.4 | 1.0 | Yes | Yes |
| 7 | Night Peak | 0.0 | 1.0 | 1.2 | Yes | Yes |
| 8 | Partial Wind Failure | 1.0 | 0.2 | 1.0 | Yes | Yes |
| 9 | Partial PV Failure | 1.0 | 1.0 | 1.0 | Yes | Yes |
| 10 | Extreme Stress | 1.0 | 0.3 | 1.4 | No | Yes |

# Chapter 5: Results and Discussion

## 5.1 Baseline Pipeline Results (Synthetic Data)

| Scenario | ABC Fitness | PSO Fitness | Load Sat % | Renew % | ABC Conv | PSO Conv |
| --- | --- | --- | --- | --- | --- | --- |
| Normal Operation | 0.3137 | 0.3137 | 100% | 72.30% | 20 | 78 |
| Peak Summer | 0.4035 | 0.4035 | 100% | 72.14% | 18 | 61 |
| Battery Failure | 0.3133 | 0.3132 | 100% | 72.12% | 29 | 8 |
| Grid Outage | 0.3137 | 0.3137 | 99.57% | 72.30% | 20 | 78 |
| Low Renewable | 0.5160 | 0.5166 | 100% | 92.77% | 17 | 3 |
| High Renewable | 0.2774 | 0.2774 | 100% | 71.91% | 27 | 37 |
| Night Peak | 0.4443 | 0.4443 | 100% | 63.51% | 27 | 9 |
| Partial Wind | 0.5204 | 0.5204 | 100% | 82.89% | 18 | 4 |
| Partial PV | 0.3704 | 0.3704 | 100% | 63.48% | 39 | 16 |
| Extreme Stress | 0.6469 | 0.6469 | 85.43% | 83.17% | 3 | 2 |

## 5.2 Real Data Pipeline Results (IEEE 33-Bus)

| Scenario | Capital ₹/day | Op Cost ₹/day | Load % | Renew % | Cable Loss kWh |
| --- | --- | --- | --- | --- | --- |
| Normal Op | ₹57,512 | ₹273,499 | 93.87% | 71.37% | 1739 |
| Peak Summer | ₹57,595 | ₹324,880 | 84.98% | 82.27% | 2104 |
| Battery Fail | ₹57,312 | ₹273,499 | 93.86% | 71.37% | 1739 |
| Grid Outage | ₹55,512 | ₹57,442 | 59.45% | 71.40% | 1739 |
| Low Renewable | ₹57,520 | ₹392,116 | 84.59% | 100% | 787 |
| High Renew | ₹27,376 | ₹266,665 | 96.94% | 62.80% | 1585 |
| Night Peak | ₹27,392 | ₹349,027 | 86.45% | 80.25% | 1446 |
| Partial Wind | ₹57,520 | ₹390,978 | 84.53% | 100% | 959 |
| Partial PV | ₹57,520 | ₹298,729 | 93.71% | 74.32% | 1447 |
| Extreme Stress | ₹65,788 | ₹516,787 | 25.80% | 100% | 1157 |

## 5.3 Key Findings

• PSO converges 3–6 iterations vs ABC 9–48 iterations — PSO is significantly faster.

• Both algorithms produce nearly identical fitness values — neither algorithm dominates in solution quality.

• Real system is capacity-constrained: 93.87% load satisfaction at full capacity, 915 kW gap at hour 19.

• Grid Outage is the most critical scenario: load satisfaction drops to 59.45% in real pipeline.

• Entropy weights reveal: Unmet Load (0.473) and Curtailment (0.362) dominate — mathematically justified, not assumed.

• Night Peak correctly sets pv_bus13_frac = 0.0, pv_bus30_frac = 0.0 — algorithm adapts automatically.

## 5.4 Capacity Recommendation Engine

Based on real pipeline analysis, the recommendation engine identifies and ranks capacity interventions:

| Recommendation | Target Scenario | Additional Capacity | Est. Capital Cost | Priority |
| --- | --- | --- | --- | --- |
| Increase Grid Import Limit | Normal (93.87%) | +1000 kW | ₹5 Cr capital | HIGH |
| Add Battery Storage | Grid Outage (59.45%) | +2000 kWh | ₹3 Cr capital | HIGH |
| Expand PV at Bus 13 | Extreme Stress (25.80%) | +500 kW | ₹2.5 Cr capital | MEDIUM |

# Chapter 6: Dashboard & Visualization

## 6.1 Web Dashboard Features

The EcoGridAI dashboard is built using Web and Plotly. It provides:

| Tab / Feature | Description |
| --- | --- |
| Scenario Analytics | KPI cards (Fitness, Cost, Load%, Renewable%, Convergence), ABC vs PSO dispatch chart, convergence curves, cost breakdown |
| 10-Scenario Benchmark | Cross-scenario bar charts, grouped comparison, fitness heatmap across all scenarios and algorithms |
| Entropy Weights Display | Visual comparison of ABC vs PSO entropy-computed weights, justification table, comparison with equal weights |
| Capacity Recommendations | Ranked expansion interventions per scenario, cost-effectiveness index, bus-level gap analysis |
| Custom Data Input | Form for bus ratings, load profile upload, field-level validation, 33-bus topology preview |
| PDF / CSV Export | One-click export of scenario results, recommendations, and charts |

# Chapter 7: Conclusions and Future Scope

## 7.1 Conclusions

EcoGridAI successfully demonstrates that swarm intelligence algorithms — ABC and PSO — can effectively optimize renewable energy microgrid dispatch on a real IEEE 33-bus distribution system. The key findings are:

• Both ABC and PSO significantly outperform random baseline across all 10 scenarios.

• PSO converges 5–10× faster than ABC, making it preferable for time-critical applications.

• Shannon Entropy Weight Method reveals that Unmet Load (47.33%) and Curtailment (36.22%) are the most important objectives — confirmed by data, not assumed.

• The real 33-bus system is capacity-constrained: 915 kW gap at hour 19 is a critical engineering finding for expansion planning.

• Grid Outage is the most vulnerable scenario: 59.45% load satisfaction highlights the need for additional islanding capacity.

• The dual-pipeline architecture (synthetic + real) enables both reproducible benchmarking and real-world application.

## 7.2 Future Scope

• Interactive Dashboard v2.0: Live optimization trigger from UI, animated convergence, multi-scenario comparison view.

• Capacity Recommendation Engine v2.0: Automated bus selection for new installation, cost-benefit analysis with payback period.

• Greenfield Planning Mode: Optimizer decides WHERE to install new assets — converting from operational to full planning tool.

• AHP Weight Method: Pairwise expert comparison for formal weight justification (complement to entropy method).

• Carbon Emission Optimization: Add CO₂ objective (diesel=2.67 kg/L, solar/wind=0) aligned with Net Zero targets.

• Multi-Microgrid Coordination: Peer-to-peer energy trading between adjacent microgrids using distributed PSO.

• Real-Time IoT Integration: Replace CSV profiles with live sensor data from smart meters.

# References

[1] Karaboga, D., & Basturk, B. (2007). A powerful and efficient algorithm for numerical function optimization: Artificial bee colony (ABC) algorithm. Journal of Global Optimization, 39(3), 459–471.

[2] Kennedy, J., & Eberhart, R. (1995). Particle swarm optimization. Proceedings of IEEE International Conference on Neural Networks, 4, 1942–1948.

[3] Zhao, B., et al. (2014). Energy management of microgrids based on a rolling horizon strategy with short-term forecasting. IEEE Transactions on Power Delivery, 30(2), 661–671.

[4] Al-Shareef, A., et al. (2016). Optimal sizing and placement of photovoltaic distributed generators in unbalanced LV networks. IEEE Trans. Smart Grid, 7(5), 2195–2203.

[5] Lee, S., & Choi, D.-H. (2019). Reinforcement learning-based energy management of smart home with rooftop solar PV, energy storage system, and home appliances. Sensors, 19(18), 3937.

[6] Rao, R.V., et al. (2012). Optimal sizing of battery energy storage systems in distribution systems using teaching-learning-based optimization. Applied Energy, 89(1), 157–163.

[7] Nguyen, T.T., & Vo, D.N. (2015). The application of one rank cuckoo search algorithm for solving economic load dispatch problems. Applied Soft Computing, 37, 763–773.